ESP32의 경우

3v3—VIN

GND—GND

D21(42)—SDA

D22(39)—SCL

3v3(8)—VIN

GND(15)—GND

D2(19)—SDA

D1(20)—SCL

WEMOS의 경우 위와 같이 연결.

사용할 코드

- MLX_Readings : MLX값을 출력

- MLXHeatCam : Processing을 GIU로 사용하여 실시간 모니터링 가능.

MLX_Readings 실행 시 기대 결과

*아두이노 환경에 맞도록 수정, ESP8266환경에서 컴파일 확인완료

Processing사용시 기대결과

MLXHeatCam.pde를 실행하여 실시간모니터링을 할 수 있음.

*MLXHeatCam 의 void setup() >> public void setting() 으로 변경하였음. 이후 디버그는 센서연결센 가능할 듯.

MLX_Readings.ino

/* MLX90640을 초기화하고 768 픽셀로부터 온도값을 출력한다. 20000 바이트 이상의 램 필요. This relies on the driver written by Melexis and can be found at: https://github.com/melexis/mlx90640-library Connect the male pins to the Teensy. The pinouts can be found here: https://www.pjrc.com/teensy/pinout.html Open the serial monitor at 9600 baud to see the output */ #include <Wire.h> #include "MLX90640_API.h" #include "MLX90640_I2C_Driver.h" const byte MLX90640_address = 0x33; //MLX의 기본 7비트 주소 #define TA_SHIFT 8 //실외에서 MLX의 default shift --실외온도인듯? static float mlx90640To[768]; paramsMLX90640 mlx90640; void setup() { Wire.begin(); Wire.setClock(400000); //12C 클럭속도 셋 Serial.begin(9600); while (!Serial); //Wait for user to open terminal Serial.println("MLX90640 IR Array Example"); if (isConnected() == false) //센서를 초기화하기 위해 먼저 isConnected () 함수를 사용하여 MLX90640과 통신 시도 { Serial.println("MLX90640 not detected at default I2C address. Please check wiring. Freezing."); while (1); } Serial.println("MLX90640 online!"); //통신성공 시 //Get device parameters - 한번만 수행하면 됨 int status; uint16_t eeMLX90640[832]; status = MLX90640_DumpEE(MLX90640_address, eeMLX90640); if (status != 0) Serial.println("Failed to load system parameters"); status = MLX90640_ExtractParameters(eeMLX90640, &mlx90640); if (status != 0) Serial.println("Parameter extraction failed"); //Once params are extracted, we can release eeMLX90640 array } void loop() { for (byte x = 0 ; x < 2 ; x++) //Read both subpages { uint16_t mlx90640Frame[834]; int status = MLX90640_GetFrameData(MLX90640_address, mlx90640Frame); if (status < 0) { Serial.print("GetFrame Error: "); Serial.println(status); } float vdd = MLX90640_GetVdd(mlx90640Frame, &mlx90640); float Ta = MLX90640_GetTa(mlx90640Frame, &mlx90640); float tr = Ta - TA_SHIFT; //센서 주변 온도(Ta)에 따른 반사 온도(tr) float emissivity = 0.95; MLX90640_CalculateTo(mlx90640Frame, &mlx90640, emissivity, tr, mlx90640To); } for (int x = 0 ; x < 10 ; x++) //출력부 { Serial.print("Pixel "); Serial.print(x); Serial.print(": "); Serial.print(mlx90640To[x], 2); Serial.print("C"); Serial.println(); } delay(1000); } //Returns true if the MLX90640 is detected on the I2C bus boolean isConnected() { Wire.beginTransmission((uint8_t)MLX90640_address); if (Wire.endTransmission() != 0) return (false); //Sensor did not ACK return (true); }

참고자료

코드설명

https://learn.sparkfun.com/tutorials/qwiic-ir-array-mlx90640-hookup-guide/all

유튜브

https://www.youtube.com/watch?v=k6qim96wB4k